HVAC and DWH Ratings Estimates

Heat Pump Ratings

| Year | Pre 1992 | 1992-2006 | 2007-2015 | 2016-2023 | After 2023 |
| --- | --- | --- | --- | --- | --- |
| HSPF | 5 | 6.8 | 7.7 | 8.2 | N/A |
| HSPF 2 | N/A | N/A | N/A | N/A | 7.5 |
| SEER | 6 | 10 | 13 | 14 | N/A |
| SEER 2 | N/A | N/A | N/A | N/A | 14.3 |

Packaged Units Manufactured after 2023 have a SEER 2 rating of 13.8

Condensing Unit Ratings (Central Air)

| Year | Pre 1992 | 1992-2006 | 2007-2015 | 2016-2023 | After 2023 |
| --- | --- | --- | --- | --- | --- |
| SEER | 9 | 10 | 13 | 14 | N/A |
| SEER 2 | N/A | N/A | N/A | N/A | 14.3 |

Condensing Units with an output of 45,000 BTU (4 tons) have a SEER 2 rating of 13.8

Electric Furnace Ratings

| KW | 5 | 10 | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- |
| BTU | 17,000 | 34,000 | 51,000 | 68,000 | 85,000 | 102,000 |

All Electric Furnaces will have an efficiency rating of 98%

Gas Furnace Ratings

| AFUE | 70% | 80% | 90% |
| --- | --- | --- | --- |
| Description | Natural Draft Furnace (No Motor on Flue) | Inducer Motor with Metal Pipe Venting the Flue Gases | Inducer Motor and PVC Pipe Venting the Flue Gases |

You can also use the SSE (%) readings from your combustion analyzer.

Electric Water Heater Ratings

| Brand | Energy Factor | First Hour | Input KW | Recovery |
| --- | --- | --- | --- | --- |
| Rheem | 0.93 | 62 | 4.5 | 98 |
| AO Smith | 0.93 | 60 | 4.5 | 98 |
| Whirlpool | 0.93 | 58 | 4.5 | 98 |
| US Craftmaster | 0.93 | 58 | 5.5 | 98 |

Other manufacturers should have similar ratings.

Gas Water Heater Ratings

| Year | Energy Factor | First Hour | Input BTUs | Recovery |
| --- | --- | --- | --- | --- |
| Pre 2020 | 0.70 | 67 | 40,000 | 75 |
| 2020 or Later | 0.90 | 86 | 40,000 | 92 |

*Note the ratings for HVACs and DWHs on this form are estimates for situations where the true ratings cannot be obtained due to age and/or deterioration of the system. If the model number and brand can be obtained, use other resources (Google) to determine these inputs. Always document the state of the current system with photos to justify your decision.*